Bottleneck-Oriented Overlap Scheduling: An Event-Driven Framework for Multi-Level BOM with δ-Overlap Guarantees

Chen-Hong

Graduate Institute of Telecommunications and Communication

National Chung Cheng University

Taiwan

leon28473787@gmail.com

Abstract

Scheduling in multi-level BOM environments is complicated by asynchronous material arrivals, partial assembly opportunities, and frequent shifts in bottleneck locations. Traditional list-based heuristics cannot react to these dynamic constraints, often leading to long or unstable schedules. This study proposes Bottleneck-Oriented Overlap Scheduling (BOS-Greedy), an event-driven framework that integrates partial-dependency activation (δ-overlap) with a dynamic bottleneck priority to guide machine assignment.

Experiments across static BOMs, δ-overlap settings, and bottleneck-deterioration scenarios reveal three consistent results:(1) BOS-Greedy matches HEFT in static environments without overlap;(2) δ-overlap provides stable, monotonic improvements in makespan;
(3) BOS-Greedy remains robust under bottleneck shifting and outperforms classic List Scheduling.Overall, BOS-Greedy offers a lightweight, interpretable, and practically deployable scheduling approach for multi-stage manufacturing systems.

Keywords:   Multi-level BOM scheduling, Dynamic bottleneck,  Event-driven scheduling,  δ-overlap,  Makespan optimization

- Introduction

Modern manufacturing systems employ multi-level Bills of Materials (BOMs) that exhibit an explicit hierarchical structure, which can be naturally represented as a Directed Acyclic Graph (DAG) with precedence constraints [1]. In this representation, nodes correspond to procurement or production operations, while edges capture the prerequisite relations and material-availability restrictions governing their initiation. Such hierarchy-driven dependency models constitute the foundational analytical framework for multi-level manufacturing scheduling problems [2, 3].

Material Requirements Planning (MRP) and Advanced MRP (AMRP) systems are commonly employed to coordinate multi-level production processes. The former determines the release times of components based on demand forecasts, inventory levels, and supply lead times, whereas the latter incorporates machine capacity, cross-facility coordination, and supply-chain delays, and is widely used in high-complexity manufacturing sectors [4, 5].

However, the widely adopted practice of partial assembly, in which certain components are processed as soon as they arrive, introduces far greater scheduling complexity than assumed in traditional MRP frameworks—particularly under practical contingencies such as machine failures and demand fluctuations [6]. When early-arriving materials allow upstream operations to begin while critical components join only in later stages, the initiation of operations is no longer equivalent to requiring all predecessors to be completed. Instead, the actual start times become governed by factors such as allowable delay tolerance, discrepancies in material arrival, and the depth of the BOM hierarchy.

In this setting, the effective critical path of the manufacturing process is often determined by the bottleneck component whose arrival is the latest, and this bottleneck may shift over time due to supply delays, processing congestion, and capacity competition [7]. Consequently, the critical path can no longer be treated as a fixed longest path structure [8], but instead evolves dynamically with the degree of allowable overlap δ.

Scheduling with precedence constraints on parallel machines is NP-hard [9]. List Scheduling provides the fundamental approximation bound of 2 - 1/m for such problems [10], forming a cornerstone of algorithmic analysis. However, prior studies have shown that local greedy decisions in multiprocessor settings may lead to timing anomalies, where increasing available resources paradoxically results in longer completion times [10].

The literature on Resource-Constrained Project Scheduling (RCPSP) indicates that when resource contention is present, the effective or precedence-feasible critical path can deviate substantially from the static longest-path structure assumed by heuristics such as the Longest Processing Time (LPT) rule [10]. This implies that the determinant of completion time is the resource-feasible critical chain, rather than the purely precedence-defined longest path [11–13]. Consequently, the traditional static longest-path perspective is insufficient to characterize the evolution of bottlenecks in practical systems where resource competition plays a central role [14, 15].

On the other hand, approximation frameworks for shop scheduling—such as the ideal schedule → random delay → flattening paradigm—provide polylogarithmic performance guarantees [16, 17], but they do not account for cross-level dependencies in multi-level BOM structures or the asynchronous arrival of materials [18].

Moreover, although the AND/OR precedence model allows an operation to start once a subset of its predecessors is completed, the formulation is largely structural in nature and does not explicitly incorporate phenomena such as lead times, hierarchical depth, or tolerable material delays. As a result, it cannot be directly applied to multi-level manufacturing settings with partial assembly practices [19].

In summary, the existing literature has yet to provide a unified scheduling framework capable of simultaneously capturing the deep hierarchical dependencies of multi-level BOM structures, the temporal evolution of dynamic bottlenecks, partial-assembly initiation, and cross-operation overlaps. Likewise, no algorithmic approach has been established that offers theoretical approximation guarantees under these combined characteristics [20, 21].

In multi-level manufacturing processes, completion time is governed by the dynamic behavior of the bottleneck chain rather than by a static longest path. This arises from heterogeneous material arrivals across BOM levels, the late insertion of critical components, the early initiation of operations under a tolerable delay δ, and shifts in bottlenecks induced by fluctuating machine loads. As argued in [22], future production scheduling and control should no longer rely on static rules; instead, they must incorporate learning-based methods to accommodate the increasingly dynamic and uncertain nature of real manufacturing environments.

Although machine learning has been widely regarded as a promising means to enhance traditional heuristics in computer systems and networking, recent systematic surveys emphasize that it cannot yet serve as a full replacement for complex decision-making algorithms. As noted in [23], most learning-based methods rely on specific data distributions or workload patterns, and their performance often degrades sharply when the operational environment shifts. Moreover, many deep models require substantial training data and costly offline exploration. From the perspective of this study, the absence of approximation guarantees implies that such models may produce unpredictable and suboptimal scheduling outcomes when confronted with intricate, atypical, or previously unseen manufacturing scenarios. This risk may translate into increased machine idle time, prolonged production cycles, and, in severe cases, failure to meet delivery deadlines.

More importantly, most machine-learning-based approaches to scheduling or routing offer limited guarantees regarding feasibility and interpretability, and lack the theoretical performance bounds that characterize classical algorithms. This makes them difficult to deploy directly in domains where safety and reliability are critical, such as scheduling, resource allocation, or bottleneck control in manufacturing. Overall, machine learning is better positioned as a predictive component or auxiliary mechanism rather than a full-scale replacement for established, structure-driven scheduling theory.

At present, no scheduling approach simultaneously:
captures such dynamic bottleneck behavior,preserves the provable structure of list scheduling,
supports overlap-based initiation with tolerance δ, and
provides deterministic approximation guarantees.However, relying solely on traditional heuristics or machine-learning models is insufficient to address the combined phenomena of cross-level dependencies, asynchronous material arrivals, and dynamic bottleneck shifts in multi-level BOM environments. The former lacks theoretical guarantees and can yield unstable schedules when bottleneck loads fluctuate, whereas the latter—despite its expressive capacity—offers limited feasibility assurance and no performance bounds, making it difficult to ensure stable and predictable scheduling outcomes in highly time-sensitive manufacturing settings. Consequently, practical production planning requires a deterministic approach that combines (1) structured decision logic, (2) verifiable approximation performance, and (3) explicit handling of overlap-based initiation and bottleneck-chain dynamics.

To this end, we propose the Bottleneck-Oriented Overlap Scheduling (BOS) method, which is built upon the following principles:
• a list-scheduling–based theoretical backbone,
• an overlap-aware initiation mechanism,
• a bottleneck-chain overlap metric for prioritization, and
• deterministic and analytically provable approximation guarantees.

- Related Work

Multi-level manufacturing scheduling involves a combination of precedence constraints, multi-machine resource contention, cross-level material arrivals, and partial-assembly initiation, making it difficult to capture using any single traditional scheduling model. This section reviews the relevant foundations of classical scheduling theory, including: (1) parallel-machine scheduling with precedence constraints and its approximation bounds, (2) dynamic bottleneck behavior in resource-constrained project scheduling, (3) structured approximation frameworks in shop scheduling, and (4) feasibility analyses of AND/OR precedence and partial-assembly models. Together, these perspectives highlight the limitations of existing approaches when confronted with multi-level BOM structures and overlap-based initiation.

2.1. Problem Setting: Multi-Level BOM, Hierarchical Precedence, and Dynamic Bottleneck

Modern manufacturing systems commonly employ multi-level BOMs to represent product structures. The hierarchical parent–child relationships among components can be naturally formalized as a DAG [24, 25]. The BOM separates the independent demand of the final product from the dependent demand of its subcomponents, and its cross-level structure dictates the sequencing of operations across production stages.

In contrast, multi-level BOMs differ fundamentally from classical parallel-machine scheduling models such as P∥prec∥Cmax. Although List Scheduling provides an approximation bound of 2 - 1/m [10], the underlying model assumes that all jobs are known at the outset and are constrained solely by logical precedence relations. These models treat jobs as abstract tasks and do not incorporate material availability, supply lead times, or cross-level arrival patterns. As a result, they represent a flat and static dependency structure that does not reflect the realities of multi-level production environments.

Multi-level BOMs, however, exhibit dependencies driven by material causality rather than simple job sequencing. High-level assemblies must wait until all subordinate components have been procured, processed, and delivered, and their feasible start times fluctuate with material arrivals, supply delays, and processing loads. In essence, the temporal structure of a multi-level BOM forms a deeply layered DAG shaped by material availability, rather than a fixed precedence graph.

Furthermore, the location of the bottleneck in a multi-level process can shift over time. In conventional precedence scheduling, the makespan is typically determined by a fixed longest path [1]. In multi-level BOM environments, however, the true limiting factor may change continuously due to variations in material arrival times, machine congestion, or queueing at downstream operations [26, 27]. Such “bottleneck shifts” imply that system behavior is no longer governed by a single dominant path, but rather exhibits the characteristics of a dynamic critical chain.

Taken together, the scheduling difficulties of multi-level BOM systems arise from the following characteristics:
• operation start times depend on the actual arrival of materials rather than predefined release times;
• delays in any component may propagate upward through the hierarchy and amplify their impact;
• the system bottleneck shifts with material flows and load distributions, rather than being determined by a single static longest path.

These properties are absent in standard parallel-machine scheduling models, indicating that multi-level BOM scheduling cannot be regarded as a subset of P∥prec∥Cmax.

2.2 Partial Kit, δ-Overlap Activation, and Asynchronous Arrival Models

Taken together, the scheduling difficulties of multi-level BOM systems arise from the following characteristics:
• operation start times depend on the actual arrival of materials rather than predefined release times;
• delays in any component may propagate upward through the hierarchy and amplify their impact;
• the system bottleneck shifts with material flows and load distributions, rather than being determined by a single static longest path.

These properties are absent in standard parallel-machine scheduling models, indicating that multi-level BOM scheduling cannot be regarded as a subset of P∥prec∥Cmax.

Another key distinction between multi-level manufacturing processes and traditional scheduling models is that operations do not always need to wait for all predecessor components to arrive before starting. In many industries—such as semiconductor packaging, automotive electronics, and assembly-based manufacturing—firms routinely adopt partial-kit strategies [28], where upstream operations begin as soon as a subset of required subcomponents is available, thereby reducing the waiting time of the final product. This practice implies that operations possess partial-start capability, deviating from the standard AND-type precedence assumption that requires all predecessors to be completed before initiation.

In this study, this phenomenon is abstracted as a tolerable overlap parameter δ. The idea is that an operation may start before all required subcomponents have arrived, as long as the missing items do not affect the portion of work that can already be executed. This behavior differs from traditional AND-precedence, which requires full satisfaction of all predecessors, and also from OR-precedence, where the completion of any predecessor is sufficient [19]. Instead, it represents a bounded intermediate form of overlap. Because δ specifies the permissible degree of early initiation, it directly shapes the feasible scheduling space and thus constitutes an essential mechanism in multi-level manufacturing processes.

Material arrivals in multi-level structures are often asynchronous, with different subcomponents reaching the production line at different times. As a result, certain stages can begin earlier while others must remain on hold. This heterogeneity in asynchronous arrivals creates a hybrid pattern in which parts of the process can overlap, whereas other parts are forced to wait.

Existing scheduling frameworks—including P∥prec∥Cmax, RCPSP [11], and AND/OR precedence models [11, 19]—generally assume strict and non-violable activation rules for predecessor constraints, making them unsuitable for describing scenarios in which operations may begin once only a subset of dependencies is satisfied. The δ-overlap mechanism, which corresponds to a “partially satisfied → permissible to start” condition, aligns with neither pure AND nor pure OR semantics. Consequently, the current literature offers no approximation algorithms that can be directly applied to multi-level BOM environments exhibiting this form of partial initiation.

Therefore, the flexibility introduced by partial-kit initiation and δ-overlap is a central reason why multi-level BOM scheduling departs fundamentally from traditional scheduling models.

2.3 Dynamic Bottlenecks and Critical-Path Deviation in Multi-Level Manufacturing

In multi-level manufacturing systems, the completion time is not determined by a single fixed longest path but by whichever segment of the production process becomes constrained at a given moment. This bottleneck shifts over time with changes in machine loads and operation sequencing, representing one of the most challenging characteristics of assembly-oriented processes.

Unlike traditional scheduling models in which bottlenecks can often be inferred in advance from the topological structure (e.g., the longest path or critical path), the sources of bottlenecks in multi-level BOM systems are far more diverse. Congestion at a particular processing stage, queueing at an upstream workstation, or even a brief delay in an intermediate component can quickly become the dominant factor slowing overall progress. This implies that bottlenecks arise as a consequence of the system’s evolving state, rather than being predetermined by a fixed path in the DAG.

More importantly, bottlenecks often shift over time. A workstation that was previously insignificant may become the new bottleneck once its load rises sharply, and the bottleneck may revert to another stage once the congestion subsides. This phenomenon of dynamic bottleneck shifting has been widely documented in manufacturing research and industrial practice [7], and is frequently cited as one of the most unpredictable factors in production systems. However, most scheduling models—including parallel-machine formulations, RCPSP, and even the classical Shifting Bottleneck framework—implicitly assume that the critical chain can be characterized through static topology or fixed machine structures, rather than accounting for rapid transitions of bottlenecks across different stages of the process.

This indicates that the critical chain in a multi-level system is not a fixed path, but one that continuously “reconfigures” over time.

A subsystem A may be the source of delay during one hour, while a different subsystem B may become the dominant bottleneck in the next. If a scheduling algorithm cannot reassess the bottleneck in real time, it is prone to making suboptimal—or even incorrect—decisions. Thus, the dynamic bottleneck phenomenon constitutes a structural challenge in its own right. It is independent of the δ -overlap mechanism or material-arrival patterns discussed in Section 2.2, and instead arises from the interaction effects inherent in multi-level processing flows. Any method intended to address multi-level BOM scheduling must therefore be capable of detecting shifts in the bottleneck position, rather than relying on static longest-path analysis.

2.4 Classical Deterministic Scheduling Algorithms: Methods and Approximation

In the study of parallel-machine scheduling with precedence constraints, deterministic approximation algorithms can be broadly classified into four major categories, each associated with a distinct analytical framework.

(1) Greedy / List Scheduling methods.
These methods determine the next job to schedule based on the current ready set, using local priority rules such as earliest start time, Longest Processing Time (LPT), or longest remaining path. Their strengths lie in their simplicity, ease of implementation, and provable performance bounds (e.g., 2 - 1/m).However, their behavior depends entirely on static topology and a fixed ready set, making them unsuitable for environments with heterogeneous arrival times or time-varying dependency conditions [10].

(2) HEFT (Heterogeneous Earliest Finish Time).
HEFT, proposed in [29], is a classical greedy algorithm for scheduling DAGs on heterogeneous processors. It consists of two main steps:

- Rank computation (upward rank): estimating global job priorities via weighted longest-path values.

- Earliest Finish Time assignment: allocating each job to the machine that yields the earliest completion time.
This structure gives HEFT strong practical stability and competitive average performance, making it a widely used baseline for DAG scheduling.

Despite its popularity, HEFT has clear limitations:

•   assumes full knowledge of all tasks (processing times, precedence relations, communication costs) at time 0;

• adheres strictly to AND-precedence (all predecessors must finish before a job can start);

•   assumes a static DAG in which the ready set does not change dynamically.

As a result, HEFT cannot model:

• cross-level dependencies and heterogeneous arrivals in multi-level BOMs;
•   δ -overlap, where partial predecessors suffice for initiation;

•   dynamic bottleneck switching;

• event-driven job release (as in BOS).

Thus, although influential in distributed computing and cloud scheduling, HEFT remains fundamentally within the realm of static AND-precedence scheduling.

(3) LP-rounding methods.

LP-based scheduling uses a relax-and-round paradigm: a linear-programming relaxation yields a fractional solution, which is then rounded to obtain a feasible schedule. These methods provide strong worst-case guarantees, including 2-approximations or PTAS results for P∥prec∥Cmax. [16, 30].

However, LP formulations assume that all jobs are known a priori and embedded in a fixed precedence structure. When job start conditions depend on material availability or δ -overlap, the feasible region of the LP relaxation no longer reflects the true system constraints.

(4) Primal–dual / dual-fitting methods.

These approaches are often applied to resource-constrained or cost-structured scheduling problems, deriving approximation ratios through the complementary slackness between primal and dual solutions. While capable of handling certain precedence-heavy problems, they likewise rely on static feasible regions and fixed activation rules, limiting their ability to capture cross-level material-arrival variability [31].

(5) Local search / neighborhood methods.

These techniques iteratively improve an existing schedule through swaps, shifts, or other local rearrangements, and are widely used in job-shop and flow-shop environments [32]. Although they often provide strong empirical performance, they typically lack rigorous theoretical guarantees and assume that the task set and underlying topology remain fixed throughout execution. Consequently, they struggle to cope with the continuously evolving ready set and bottleneck shifts present in multi-level BOM systems.

These classes of methods have led to several classical approximation guarantees and collectively form the theoretical foundation of deterministic scheduling.

Classical deterministic approximation results have emerged from these methodological directions. The 2 - 1/m bound established by Graham’s List Scheduling remains the most fundamental and widely cited guarantee for parallel-machine scheduling with precedence constraints, forming the basis for much subsequent analysis. LP-based approaches, exemplified by the work of Hall and Shmoys, demonstrate that LP relaxations combined with rounding can achieve 2-approximations and, under certain structural conditions, even PTAS results. In more complex settings such as job-shop scheduling, the polylogarithmic approximation of Shmoys, Stein, and Wein represents a major breakthrough, though it still relies on fixed dependency structures and static job sets. Additional deterministic bounds—such as those by Potts and Van Wassenhove on batching, release dates, and machine-dependent constraints—typically fall within the 2–3 approximation range, but similarly presuppose full information at time zero and a fixed DAG representation.These results establish the theoretical bounds of deterministic scheduling in static settings, yet they still share several inherent limitations.

Although these methods provide strong theoretical foundations for static precedence scheduling, their modeling assumptions prevent direct application to multi-level BOM environments. A fundamental limitation is that they can only accommodate AND-type precedence, requiring all predecessors to be completed before a task may start; thus, they cannot express δ -overlap or partial-kit initiation. Even the AND/OR precedence formulation of Möhring relaxes this requirement only partially and does not yield an approximation framework capable of handling overlap-based activation. Moreover, these approaches presuppose that all tasks are known and schedulable at time zero—an assumption shared by LP-based algorithms, List Scheduling, and local-search heuristics—whereas in BOM-driven systems, task availability is determined by the actual arrival of materials rather than predefined release times. They also lack the ability to capture the structural complexity of multi-level BOMs, including cross-level delay propagation, heterogeneous procurement or processing lead times, and inconsistent release patterns, none of which can be represented within the standard P∥prec∥Cmax.framework. Finally, these methods operate on a fixed precedence graph and therefore assume a static critical path, making them incapable of tracking or controlling dynamic bottleneck shifts induced by material-arrival order, machine-load variability, localized congestion, or early initiation enabled by δ -overlap. Such fixed-topology assumptions undermine the applicability of deterministic approximation guarantees in multi-level manufacturing settings.

In summary, traditional deterministic approximation algorithms are highly effective within static and fully known precedence-scheduling settings, yet their structural assumptions are fundamentally incompatible with the dynamic nature of multi-level BOM environments. They cannot represent partial-dependency activation such as δ -overlap, nor can they accommodate heterogeneous and asynchronous material arrivals that determine when tasks actually become executable. Likewise, they are unable to track rapid shifts in bottleneck locations across different stages or model the propagation of delays through hierarchical layers. Consequently, despite their well-developed theoretical foundations, these methods operate in a problem space fundamentally distinct from the requirements of multi-level BOM scheduling.

Consequently, recent studies have increasingly explored learning-to-schedule and other machine learning–based approaches to address dynamic environments, aiming to overcome the limitations of deterministic methods in multi-level processes. However, machine learning–based scheduling also carries significant drawbacks, particularly regarding feasibility guarantees, interpretability, and worst-case performance bounds.

2.5 Machine-Learning-Based Scheduling: Strengths, Weaknesses, and Instability Issues

Recent research has explored several directions in machine learning–assisted scheduling. Reinforcement learning methods, such as DeepRM and Decima, train agents to learn dispatching policies and job-selection strategies directly through interaction with the environment [33, 34]. Other studies adopt supervised learning to infer dispatching rules from historical scheduling data, enabling models to predict job priorities under observed conditions [35]. More broadly, learning-to-schedule frameworks formulate scheduling as a sequential decision-making problem, where deep models generate the next scheduling action based on the current system state. Despite their conceptual flexibility, these approaches typically assume a fixed DAG topology and a stable machine environment, and they rely heavily on extensive offline training to achieve acceptable performance.

The primary strengths of machine learning in scheduling arise from its ability to recognize patterns, adapt to recurring workload structures, and provide predictive signals. Its pattern-recognition capability allows models to extract latent dispatching rules from large collections of historical data. Its adaptivity enables the learned behavior to generalize well in systems where workload characteristics exhibit stable or repetitive structure. Moreover, its predictive capacity—such as forecasting processing times, machine loads, or delay risks—can offer valuable auxiliary information for scheduling decisions. For these reasons, machine learning is particularly well suited to serve as a predictive module or a decision-support component rather than a standalone scheduling mechanism.

Despite these advantages, replacing algorithmic scheduling with machine learning–based methods remains fundamentally limited. As a data-driven approach, machine learning provides no performance guarantees and thus lacks the approximation bounds required in high-reliability production settings. Prior studies further show that models trained through reinforcement learning or supervised learning can degrade catastrophically when workload characteristics or product structures deviate even slightly from the training distribution. Moreover, feasibility is not inherently enforced: machine learning models frequently generate schedules that violate precedence relations or material-availability constraints, a well-documented issue in both academic prototypes and industrial deployments. Beyond feasibility, machine learning approaches struggle to capture the causal dependencies inherent in multi-level BOM systems; they typically learn priority rules over tasks, whereas BOM constraints arise from material causality rather than observable statistical patterns. Finally, these methods are unable to model or respond to dynamic bottleneck shifting, leading to unstable or inconsistent scheduling behavior under multi-stage, load-dependent bottleneck migration. For these reasons, machine learning is better suited as an auxiliary component—such as a predictive or estimation module—rather than the core mechanism for addressing scheduling problems that combine multi-level BOM structures, δ -overlap initiation, and dynamic bottleneck behavior.

2.6 Comparative Gap Analysis: Why Existing Methods Cannot Solve BOM + δ-Overlap

The comparison in Table 1 summarizes the capability of different scheduling approaches with respect to multi-level BOM structures, δ -overlap initiation, bottleneck dynamics, approximation guarantees, and feasibility.

Table 1. Comparison of scheduling methods

| Method | Multi-level BOM | δ -Overlap | Bottleneck Dynamics | Approx. Guarantee | Feasibility |
| --- | --- | --- | --- | --- | --- |
| Greedy / List Scheduling | ✘ | ✘ | ✘ | ✔ | ✔ |
| LP Rounding | ✘ | ✘ | ✘ | ✔ | ✔ |
| Primal–Dual / Local Search | ✘ | ✘ | ✘ | ✔ | ✔ |
| Machine Learning Scheduling | ✘ | ✘ (no guarantees) | ✘ (unstable) | ✘ | ✘ |
| BOS (this study) | ✔ | ✔ | ✔ | ✔ (deterministic) | ✔ |

2.7 Positioning of BOS: Contributions Over Existing Methods

The preceding review indicates that neither classical deterministic scheduling algorithms—such as list scheduling, LP rounding, primal–dual methods, and local search—nor recent machine learning–based scheduling approaches can simultaneously address the three tightly coupled characteristics observed in practice: multi-level BOM structures, δ -overlap initiation, and dynamic bottleneck behavior. To overcome these limitations, this study proposes the Bottleneck-Oriented Overlap Scheduling (BOS) framework, whose core contributions can be summarized as follows:

The BOS framework offers several contributions that directly address the limitations of existing scheduling paradigms. First, it provides a deterministic and fully interpretable scheduling rule built upon an analyzable list-scheduling backbone, avoiding the opacity and feasibility risks associated with data-driven black-box models. Second, BOS introduces an approximation-oriented analytical structure, using the residual bottleneck-chain length Di to extend the traditional longest-path concept into dynamic environments. This preserves the provable scheduling structure of list scheduling while exhibiting predictable behavior even under adversarial ordering. Third, the method natively captures the hierarchical and causal nature of multi-level BOM systems: instead of modeling pure task precedence, BOS directly reflects material causality, cross-layer dependencies, and heterogeneous arrival times—elements not representable within P∥prec∥Cmax.or RCPSP formulations. Fourth, BOS explicitly incorporates δ -overlap activation, enabling operations to start legally under partially satisfied dependencies and thus reflecting the partial-kit practices common in real manufacturing. Fifth, because BOS operates in an event-driven manner, it continuously recomputes the bottleneck chain after each completion, arrival, or resource-release event, allowing it to track and react to dynamic bottleneck shifting rather than relying on a static critical path. Finally, BOS bridges the gap between classical approximation algorithms and industrial scheduling heuristics: it is more structured, predictable, and analytically grounded than traditional heuristics; more stable, interpretable, and feasibility-preserving than machine-learning-based scheduling; and more aligned with material-driven manufacturing logic than standard precedence-based models. As a result, BOS stands as one of the few scheduling frameworks capable of simultaneously handling multi-level BOM structures, δ -overlap initiation, and dynamic bottleneck behavior.

- BOS-Greedy Scheduling Algorithm

In the preceding chapter, we showed that multi-level manufacturing environments exhibit deeply intertwined characteristics—multi-layered dependencies, δ\deltaδ-overlap initiation, and dynamic bottleneck behavior—that cannot be fully addressed by traditional precedence-based scheduling models or by existing deterministic and machine learning–based approaches. This chapter introduces the core structure of the proposed Bottleneck-Oriented Overlap Scheduling (BOS) framework, using its lightest and most representative variant, BOS-Greedy, as the foundation. The design objective of BOS-Greedy is to preserve analytical tractability and interpretability while capturing material causality, overlap-based activation, and the evolving bottleneck chain in multi-level processes, thereby enabling a scheduling mechanism that updates in real time under dynamic events and maintains theoretically bounded behavior.

Section 3.1 begins with the formal problem definition and subsequently details the scheduling model, event-driven execution flow, and core priority metric underlying BOS-Greedy, illustrating how the method preserves feasibility and approximation-like properties within the complex temporal structure of multi-level BOM environments.

3.1 Algorithm Overview

BOS-Greedy is an event-driven scheduling approach designed specifically for multi-level BOM environments with dynamic bottlenecks. Unlike traditional parallel-machine scheduling methods, BOS-Greedy does not rely on static topology or a fixed critical path. Instead, it continually updates task priorities and reallocates machines based on the real-time state of the manufacturing process. The central idea is to, at every event, prioritize the task belonging to the bottleneck chain that is most likely to extend the overall completion time. The algorithm is built around three key design principles:

1. Dynamic bottleneck index Di (Dynamic Criticality)

The priority of each task is determined by its remaining critical-chain depth. The value of Di is updated as predecessors finish, materials arrive, δ-overlap becomes activated, or machine states change, allowing it to reflect the continual movement of bottlenecks in multi-level manufacturing systems.

2. Event-driven scheduling architecture

The schedule does not progress continuously along the time axis; instead, it is triggered only when system events occur:

• task completion,
• activation of a task due to material arrival or partial-kit readiness,
• machine release or machine-state updates.

After each event, BOS-Greedy re-evaluates only the affected tasks and updates the priority queue, avoiding the need to recompute the entire schedule and thereby improving scalability.

3. Bottleneck-oriented machine allocation

BOS-Greedy maintains the executable task set R using a max-heap and allocates machines to tasks with the largest Di. When necessary, the algorithm performs legal preemption on tasks with lower Divalues, ensuring that limited resources remain focused on the most critical chain and preventing bottleneck deterioration. Operating over a set of parallel machines, the algorithm updates machine assignments based on the priority queue after each system event. The overall procedure can be abstracted into four steps:

(1) compute Difor all tasks;

(2) build the executable task pool R and max-heap HHH;

(3) updateDi, R, and H whenever an event occurs;

(4) assign tasks to available machines M in the order specified by H until all tasks are completed.

This bottleneck-oriented dynamic design enables BOS-Greedy to maintain stable and consistent decision logic in multi-level, asynchronous, and δ-overlap environments, while preserving analytical tractability and interpretability. It is therefore well suited for scheduling and real-time control in dynamic multi-stage manufacturing systems.

Given this event-driven framework, the core behavior of BOS-Greedy depends entirely on the formulation of the task-level priority measure. Whenever materials arrive, predecessors complete, or δ-overlap conditions activate a task earlier than expected, the algorithm must immediately determine which processing chain is most likely to become the next bottleneck. Before presenting the detailed event-driven procedure, the next section introduces the central element of BOS-Greedy—the dynamic bottleneck indexDi—which quantifies each task’s potential contribution to the overall completion time and is updated continuously as system events unfold. The following subsection formalizes the definition, update rules, and algorithmic role of this index.

Algorithm 1 . BOS-Greedy: Summary of Algorithmic Steps

1.Compute the dynamic priority Di for each task, defined as its remaining critical-chain depth. 2. Construct the executable task pool R and organize it using a max-heap–based priority queue. 3. Upon each execution or completion event: •if a new task becomes executable, insert it into the heap; • if a task completes or is preempted, remove it from the heap; • update machine assignments and perform preemption when necessary. 4. Repeat the above steps until all tasks have been completed.

Algorithm 2 . BOS-Greedy Algorithm: Event-Driven Procedure

1. Initialization: Construct the initial executable task set R, consisting of all tasks whose release times and δ -overlap activation conditions have been satisfied. 2. Priority Queue Construction: Build a max-heap H over R, using the remaining critical-chain length Dias the priority key. 3. Event-Driven Scheduling Loop: While there remain unfinished tasks in the system, repeat the following steps: 3.1 Wait for the next event e. Possible events include: – task completion; – activation of a task due to predecessor completion or δ\deltaδ-overlap; – machine release or recovery; – any other system event that may affect task feasibility. 3.2 Update task states: – recompute Difor tasks affected by the event; – update the executable set R by inserting or removing tasks accordingly. 3.3 Update the priority queue H: – push newly executable tasks into H; – pop tasks that have completed or been preempted; – adjust heap order to reflect updated Divalues. 3.4 Machine allocation: – assign idle machines to tasks in descending order of Difrom H; – when necessary, legally preempt lower-priority tasks and reinsert them into the appropriate position in the priority queue. 4. Termination: The algorithm terminates once all tasks have been completed.

3.2 Dynamic Priority Di：Remaining Critical-Path Length

In BOS-Greedy, every scheduling decision is based on comparing task priorities. To capture the characteristics of multi-level BOM structures and dynamic bottlenecks, this study does not rely on simple metrics such as processing time, earliest start time, or slack. Instead, each task iii is assigned a dynamic bottleneck index Di which estimates how much the overall completion time may be extended if system resources do not prioritize task i at the current moment. The design of Di follows three fundamental principles:

- it must account for the remaining processing time of the task itself;

- it must incorporate the residual load of the entire successor chain rather than treating each task in isolation;

- it must reflect the amount of “overlap slack’’ enabled by δ -overlap and material-arrival conditions—that is, the portion of future time that can still be compressed.

For clarity, we first introduce the notion of remaining critical-chain length before formally defining δ.

Consider the task set J and the directed acyclic graph G=J,E induced by the multi-level BOM structure.
For each task i, we define:

• pit: the remaining effective processing time of task i at time t (equal to the original processing time if the task has not yet started, or the unfinished portion if it is already in progress).

• Succi: the set of all successor tasks reachable from i in G—that is, all unfinished tasks that lie downstream of iii in the multi-level BOM structure.

For a task i and any feasible path

P=(i,j1,j2,…,k)

originating at iii and ending at some leaf node k, we define the remaining processing length of path P at time t as

LP,t=pit+pj1t+pj2t+⋯+pkt,	(1)

which represents the total processing time required, from the current moment onward, to complete all unfinished tasks along that path.

If we ignore δ -overlap and parallel-machine effects, the remaining critical-chain length of task iii can be defined as the maximum remaining processing length among all paths originating from i:

CPit = maxP starting from iLP,t.	(2)

This quantity CPitcharacterizes the processing time still required—absent any further overlap or parallel compression—to complete the heaviest successor chain extending from task iii to the end of the product’s assembly process.

In practical multi-level manufacturing processes, certain operations may begin within a tolerable overlap δ. For example, an assembly task may start processing available subcomponents even if a few critical parts have not yet arrived. This implies that the critical-chain measure CPitin (2) is a conservative upper bound, as it does not yet incorporate the compression effects enabled by such overlap.

To incorporate this “overlap slack’’ into the analysis, we define for each path P an overlap discount OverlapP,t, which represents the total amount of processing time along the path that can be compressed through δ -overlap. The underlying idea is as follows:

• If two adjacent operations can overlap within a duration δ, then under stable conditions that segment of the chain can theoretically reduce its waiting time by up to δ.

• If the upstream operation has already begun early or is near completion, the actual compressible amount may exceed δ.

• If material delays or machine congestion occur, the available overlap slack may be partially offset or may disappear entirely.

Without requiring a fully formalized set of conditions, the effective critical-chain length of a path P at time t can be defined as:

EffP,t = LP,t - OverlapP,t	(3)

That is, the effective critical-chain length of path P at time t is given by its remaining processing length minus the portion of time that can be compressed through overlap. Next, by taking the maximum over all feasible paths originating from task i, we define

Dit = maxP starting from iEffP,t	(4)

This quantity Ditis the dynamic bottleneck index employed by BOS-Greedy.
Intuitively, it represents the longest unavoidable remaining critical-chain length starting from task i, after accounting for the overlap possibilities currently available in the system. A larger value of Dit indicates a greater risk that the overall completion time will be extended if the chain containing task i is not prioritized at the present moment. Consequently, BOS-Greedy allocates machine resources first to tasks with the largest Dit.

Since Ditdepends on the current system state, it must be updated locally when events occur rather than recomputed over the entire graph at every time step. BOS-Greedy updates only the Ditvalues affected by the following types of events:

- Task completion or progress updates
When a task j completes, the remaining processing times and available overlap slack of its successor tasks change accordingly. In this case, the corresponding CPitand Dit values must be updated upward along all predecessor nodes of j.

- Material arrival or activation through δ -overlap
When an assembly task becomes executable earlier due to partial-kit readiness, the overlap discount OverlapP,t along the affected paths increases, indicating additional compressible time. Only the Dit values in the vicinity of the activated task need to be updated, without recomputing the entire graph.

- Machine-state changes (release, failure, recovery)
Although machine availability does not explicitly appear in the formula for Dit machine congestion affects the realizable amount of overlap. In the implementation of BOS-Greedy, a heavily congested machine can be treated as a factor that reduces the overlap discount, prompting adjustments to OverlapP,t along the relevant paths.

- Arrival of new jobs or insertion of new tasks
In dynamic environments where new tasks are added to the existing BOM structure, the system only needs to recompute Dit for the portion of the critical chain affected by the new task, rather than rescheduling all existing tasks.

Through this event-driven and locally updated mechanism, BOS-Greedy keeps the dynamic bottleneck index Di aligned with the evolving system state. This approach preserves scalability while ensuring that each scheduling decision is guided by a meaningful and up-to-date assessment of bottleneck risk.

3.3 Event-Driven Scheduling Framework

Traditional parallel-machine scheduling methods typically follow a time-driven paradigm, in which the system updates all task states, checks machine availability, and re-evaluates dispatching decisions at every time increment. However, in multi-level BOM environments, task feasibility depends on a combination of factors—including material arrivals, predecessor completion, δ -overlap activation, and machine-state changes. Recomputing the entire schedule at fixed time intervals would result in substantial computational overhead and would fail to capture the fine-grained state transitions inherent to multi-stage manufacturing processes.

For this reason, BOS-Greedy adopts an event-driven scheduling paradigm. The central idea is that decisions are updated only when the system state actually changes. In other words, scheduling does not progress continuously along the time axis but is triggered exclusively by the occurrence of specific events.

In multi-level manufacturing systems, several types of events may alter the system state:

- Completion Event
When a task finishes, its successor operations may become executable, and the remaining load along certain paths may decrease, potentially causing the bottleneck position to shift.

- Activation Event
In a BOM structure, certain operations can start as soon as a subset of required components has arrived or when δ -overlap conditions are satisfied. Such activation events introduce new tasks into the executable set and necessitate updates to the priority ordering.

- Machine Update Event
Changes in machine availability—such as machine release, maintenance, or recovery—affect the realizable degree of overlap along bottleneck chains. In particular, machine failures increase the effective load on affected paths, requiring immediate priority adjustments.

- Arrival of New Tasks (e.g., dynamic orders)
In dynamic scheduling environments, new orders, new production items, or new branches of the BOM may be introduced. The system must recompute priorities only within the region impacted by these additions.

Whenever any of these events occurs, BOS-Greedy proceeds to the next step in the scheduling procedure.

Compared with time-driven scheduling, the event-driven approach offers several notable advantages. First, it avoids unnecessary recomputation by updating decisions only when the system state changes, thereby improving scalability and eliminating large amounts of idle evaluation. Second, it captures the inherently asynchronous nature of multi-level BOM structures and δ -overlap activation, where the timing of material arrivals is often the dominant source of complexity; event-driven scheduling responds to these changes precisely when they occur. Third, it enables continuous tracking of dynamic bottlenecks, whose locations may shift due to processing delays, material-arrival variability, or fluctuations in machine load. By recalibrating task priorities after each event, BOS-Greedy maintains consistent and coherent decision-making throughout the evolution of the manufacturing process.

Unlike classical list scheduling, which fixes a single ordering and executes it to completion, BOS-Greedy recalibrates priorities after every event, enabling decisions to adapt continuously to the evolving system state. In contrast to RCPSP-based approaches that rely on static precedence relations and rigid resource constraints, BOS-Greedy accommodates δ-overlap and inherently dynamic bottleneck behavior. Furthermore, unlike machine learning–based scheduling, its decisions remain interpretable, predictable, and analytically bounded, offering a level of transparency and theoretical rigor essential for high-reliability manufacturing environments.

3.4 δ-Overlap Activation Rule

A defining property of multi-level BOM systems is that many operations can begin without waiting for all predecessor components to arrive; partial work can proceed as long as a sufficient subset of materials is available. This practice—commonly observed in semiconductor packaging, automotive electronics, and assembly-based manufacturing—is often referred to as partial kit or early-stage preprocessing. BOS-Greedy formalizes this flexibility through the notion of a δ-overlap activation rule.

The basic idea of δ -overlap is as follows. If a subset of a task’s predecessors has already been completed and the remaining predecessors only delay downstream work without affecting the portion that can be processed immediately, then the task may begin up to δ time units earlier than full material readiness. A larger δ corresponds to a greater tolerance for early initiation. In other words, δ characterizes the acceptable degree of incompleteness in predecessor availability.

For a task i, if its predecessors or required materials are not fully available but the following conditions hold, BOS-Greedy deems task iii eligible for early execution:

- The missing predecessors affect only downstream processing and do not impede the portion of work that can be initiated immediately.
For example, an assembly step may require five components in total, but if three are already available and the remaining two are used only in a later sub-stage, the task can begin.

- The worst-case delay caused by the missing predecessors does not exceed δ.
If a delayed predecessor would extend the downstream chain by no more than δ, the early start is permitted.

- Early initiation does not introduce logical inconsistencies—for instance, the operation must not require immediate use of materials that have not yet arrived.

When these conditions are satisfied, task i triggers a

→δ-overlap Activation Event.

and is immediately inserted into the executable task set.

Once a task i is activated early through δ-overlap, it exerts three immediate effects on the scheduling process. First, it enters the executable set and is inserted into the max-heap H, where it will be considered in the next event cycle according to its updated dynamic bottleneck index Di. Second, its activation increases the available overlap slack along the associated paths, reducing Di and, to a lesser extent, the indices of its predecessors. This reflects the fact that early initiation introduces additional potential for compressing the overall makespan, thereby lowering the bottleneck risk of the corresponding chain. Third, the activation may shift the location of the system’s bottleneck: operations that were previously delayed by unmet predecessors may progress earlier, causing downstream tasks—or even other departments—to become the new limiting stages. During the event-update phase, BOS-Greedy automatically recomputes the relevant Di values to capture this shift.

The notion of δ -overlap is not merely a relaxation that permits early starts; it is a structural necessity inherent to multi-level manufacturing systems. Material arrivals in a BOM are inherently asynchronous, and many processing steps can legitimately begin before all components are available. Enforcing strict AND-type dependencies often leads to unnecessary waiting and idle machines, while prohibiting overlap entirely would cause real-world production lines to override the schedule—“force starting” tasks in ways that invalidate the model. For this reason, δ-overlap constitutes a fundamental element that differentiates BOS-Greedy from traditional scheduling formulations.

Conventional RCPSP formulations allow a task to start only after all of its predecessors have been completed and therefore cannot represent partial activation. Although AND/OR precedence structures introduce some flexibility by permitting alternative prerequisite sets, they still cannot capture situations in which a small subset of missing components is tolerable and the operation may legitimately begin. The notion of δ -overlap thus represents a distinct form of tolerable partial dependency that lies between strict AND and OR relationships and arises naturally in real manufacturing environments.

3.5 Machine Allocation & Preemption Logic

After updating task states and reconstructing the priority queue, the central operation of BOS-Greedy is to assign available machines to the appropriate tasks. Because the bottleneck in a multi-level BOM environment may shift after each event, machine allocation is not merely a task-selection step but a critical decision that shapes the evolution of the overall bottleneck chain. BOS-Greedy adopts a bottleneck-oriented resource-allocation strategy: whenever a machine becomes available, it is assigned to the task with the largestDi; if necessary, tasks with lower Di values may be preempted so that limited processing capacity can be concentrated on the chain currently most critical to the system’s completion time.

After each event, the max-heap H reflects the updated priority ordering of all executable tasks. BOS-Greedy then iterates over the available machines and performs the following steps. First, the task i with the largest Di is extracted from H, as it represents the operation whose chain is currently most likely to extend the overall makespan. If an idle machine is available, task i is assigned immediately—without delay, waiting, or additional ordering computations. If all machines are occupied, BOS-Greedy compares iii with the tasks currently running: if a task j on some machine has a significantly smaller Di and can be legally preempted (according to δ -overlap conditions or the nature of the operation), then preemption is performed and the machine is reassigned to task i. If no task is eligible for preemption, iii remains in H and waits for the next event. The purpose of this procedure is to ensure that processing capacity is consistently directed toward the operation that contributes most critically to the extension of the bottleneck chain, thereby preventing bottleneck propagation.

Not all tasks are eligible for interruption. BOS-Greedy permits preemption only when all of the following conditions are satisfied. First, the task j under consideration must be safely pausable; its processing type must allow suspension and later resumption, as in partial processing, preparatory assembly steps, or multi-stage operations. Second, preemption must not cause the completion time of j to become the new bottleneck—if the remaining critical-chain length of j is itself substantial, interrupting it would be counterproductive. Third, the reduction in bottleneck pressure achieved by prioritizing task iii must exceed the increase caused by delaying j; in other words, preemption must yield a net improvement to the system’s bottleneck profile. Finally, preemption must respect the δ -overlap constraints: if suspending j would prevent its successors from remaining activatable within the allowable δ -tolerance, preemption is disallowed. These conditions ensure that BOS-Greedy avoids excessive or indiscriminate preemption, thereby preventing instability in the scheduling process.

When preemption occurs, the system immediately performs the following actions. First, the remaining processing time and current state of task j are updated; task j is marked as paused and reinserted into the max-heap H. Second, the assignment state of task iii is updated, and iii begins execution on the corresponding machine with its new start time recorded. Third, the dynamic bottleneck indices of the affected tasks, including Di and Dj, are locally recomputed to reflect any changes in the bottleneck chain caused by the preemption. Finally, the event concludes, and the system waits for the next event to trigger another round of updates. This “event trigger → state update → heap adjustment → preemption’’ mechanism enables BOS-Greedy to effectively track and respond to shifts in the bottleneck structure.

Unlike traditional parallel-machine scheduling, BOS-Greedy does not aim to maximize the local utilization of individual machines. Instead, its focus lies on a higher-level objective: preventing the emergence of a longer critical chain. In other words, the allocation strategy is not concerned with whether each machine is fully loaded, but rather with whether postponing a particular task might delay the overall completion time of the BOM. This design philosophy makes the algorithm particularly well suited to environments characterized by multi-level BOM structures and dynamically shifting bottlenecks.

In contrast to classical list scheduling, BOS-Greedy does not rely on a single global ordering executed to completion; instead, it reallocates machines dynamically in response to system events. Unlike RCPSP formulations, its decisions are not driven solely by resource constraints but by the need to continuously steer processing capacity toward the direction of the evolving bottleneck. Moreover, unlike machine learning–based scheduling, BOS-Greedy maintains a fully interpretable and predictable preemption logic, ensuring consistent decision-making without dependence on opaque black-box models.

The following pseudocode presents the complete event-driven workflow of BOS-Greedy. The algorithm relies on three primary data structures:

• the executable task set R, containing all tasks whose material requirements or δ-overlap conditions have been satisfied;
• the max-heap H, ordered by the dynamic bottleneck index Di;
• the machine set M, representing all available or currently running machines.

Algorithm.3 BOS-Greedy: Event-Driven Pseudocode

Input: - Task set T - Machine set M - Precedence graph (BOM) - Allowed δ-overlap value - Material release times release_timei - Processing times pi Output: - Scheduling results for all tasks (start time, finish time, assigned machine) Procedure BOS_Greedy(): - Initialization: R ← all tasks satisfying release-time and δ -overlap activation conditions For each task i∈R, compute its dynamic bottleneck index Di Push all i∈R into the max-heap H Mark all machines as idle While there exist unfinished tasks: 2.1 Wait for the next event e, where ∈{task completion, task activation, machine release , material arrival} 2.2 Update task states according to the event: - If a task j completes: mark j as finished and update the predecessor/activation state of its successors - If a task k becomes executable: insert k into R 2.3 Update dynamic bottleneck indices: For every affected task u: recompute Du 2.4 Update the max-heap H: - For newly executable tasks: push them into H - For completed or preempted tasks: remove them from H - If D[u] changes: adjust the heap position of u (heapify) 2.5 Machine allocation: For each idle machine m ∈M: - If H is not empty: i←popH assign task iii to machine mmm mark task i as running 2.6 Preemption check: If the highest-priority task i* in H has a substantially larger Di* than some currently running task j: - If j is legally preemptible: pause task j, push j back into H reassign the machine to task i* mark i* as running 3. When all tasks are completed: return the full scheduling results

3.7 Computational Complexity

This section analyzes the computational complexity of the BOS-Greedy algorithm for multi-level BOM scheduling. We adopt the following notation:

• n: number of jobs/operations in the system
• E: number of precedence edges
• m: number of parallel machines
• the number of events—comprising task activations, task completions, and machine-state changes—is on the same order as n and E

The computational cost of BOS-Greedy arises primarily from three components:

(1) initialization of the graph and priority metrics;
(2) state updates triggered by events;
(3) machine allocation and possible preemption decisions.

Before scheduling begins, the algorithm performs the following steps:

- Scan the DAG induced by the BOM and construct the predecessor and successor sets for each operation.
This corresponds to reading the entire graph, yielding a complexity of On+E.

- Compute the initial value of Di (remaining critical-chain length) for each operation based on the topological structure.
Using a dynamic programming pass from terminal nodes backward—taking, for each node, the maximum over its successors plus its own processing time—the total computation can also be completed in On+E.

- Insert all operations satisfying material-release and δ-overlap activation conditions into the executable set R, and construct the max-heap HHH using Di as the key.
Building the heap has a cost of OR, which in the worst case is On.

Thus, the total computational cost of the initialization phase is On+E.

In the main scheduling loop, BOS-Greedy performs updates whenever a system event occurs. For each event, the processing steps include the following:

- Updating task states and Di.
When a task completes, all of its successors have their “number of completed predecessors’’ updated.
If enough predecessors have finished to satisfy the δ-overlap activation condition, the corresponding task is inserted into R.
Updates to Di are confined to locally affected nodes (e.g., the task itself and a short upstream segment of its chain). Under a reasonable implementation, each edge is updated only a constant number of times; thus, the total cost of updating all Divalues across the graph is OE.

- Updating the max-heap H.
Newly executable tasks may be pushed into H multiple times throughout the schedule (and again upon resumption if preemption is allowed). Each push operation costs Olog n.
Completed or preempted tasks are removed from H, corresponding to pop or decrease-key operations, also costing Olog n.
Since each task is inserted and removed a bounded number of times during its lifetime, the total number of heap operations is on the order of On, giving an overall heap-maintenance cost of Onlogn.

- Number of events.
Each task has at least one activation event and one completion event, plus a small number of additional events stemming from machine-state changes such as preemption or recovery.
Thus, the total number of events grows as a constant multiple of n rather than increasing continuously with time.

Combining these observations, the cumulative cost of state updates and heap operations over the entire scheduling horizon is:

- State and Di updates: OE

- Priority-queue operations: Onlogn

After each event is processed, BOS-Greedy assigns tasks to the idle machines in M according to the ordering in the max-heap H This step consists of the following components.
First, extracting the task with the largest Di from H requires Olog n. Second, the algorithm checks whether preemption is necessary by comparing this task with those currently running. If each machine needs to be compared only once with the candidate task, the preemption check per event is Om, where m is typically much smaller than n
Third, any preempted or newly activated tasks are updated in H, which again involves either a push or heap adjustment and therefore remains Ologn.

Since each event triggers only a limited number of machine-allocation and preemption checks, and the total number of events is On, the overall computational cost associated with machine allocation and preemption decisions is

Onlogn+nm.

In most practical settings, mmm is a constant or significantly smaller than n, and thus this component does not dominate the total complexity.

Combining the three components discussed above, the overall computational complexity of BOS-Greedy can be expressed as follows:
• Initialization and initial computation of Di: On+E
• Event-driven updates and heap operations: OE+nlognO
• Machine allocation and preemption decisions: Onlogn+nm

When n ≫m and when E is on the same order as n or slightly larger, the dominant term becomes

On+Elog n.

This indicates that even under multi-level BOM structures and dynamically shifting bottlenecks, BOS-Greedy preserves a polynomial-time complexity on the same order as classical list scheduling, making it well suited for practical deployment in large-scale multi-stage manufacturing systems.

3.8 Performance Guarantee

Let
• P: the total processing time of all tasks (the sum of processing times),
• m: the number of machines,
• Lδ: the effective critical-path length after accounting for δ-overlap
(when no overlap is allowed, Lδ=L, the traditional critical path).

Let CmaxBG denote the makespan produced by BOS-Greedy.
We obtain the following upper bound:

Theorem 1. Upper Bound on the Makespan of BOS-Greedy

For any scheduling instance, the following inequality holds:

CmaxBG≤Pm+1-1mLδ.

When m = 1, the bound reduces to

CmaxBG =Lδ,

which is necessarily optimal.

When m is large, the bound approaches Lδ, implying that the dominant factor becomes the bottleneck path.

This result naturally generalizes two classical regimes:

- δ= 0 (no overlap).
The bound collapses to

Pm+1-1mL,

which is exactly Graham’s classical list-scheduling bound.

- Large δ (high overlap allowed).
The quantity Lδ becomes significantly smaller than L,
yielding a strictly tighter (better) bound.

In other words, BOS-Greedy provides a δ -overlap generalization of the classical list-scheduling upper bound.

The core idea of the analysis is that the total schedule length T (i.e., CmaxBG) can be decomposed into:

the total busy time of all machines (equal to the total workload P), and

the idle time of all machines, denoted I.

Thus,

m⋅T=P+I⇒T=Pm+Im.

To prove the theorem, it suffices to show that

I≤m-1Lδ.

The proof proceeds by decomposing the total schedule length into its busy and idle components. First, when all machines are actively processing tasks, the system expends mmm units of processing capacity per unit time; therefore, the total amount of fully utilized time can never exceed the load lower bound P/m. Any remaining time units in which the machines are not fully utilized must arise solely from blocking on the residual bottleneck chain. Under BOS-Greedy, a machine idles only when no eligible task can be started, which implies that some unfinished task at the end of a critical chain is preventing downstream work from becoming executable. Since Lδ denotes the longest chain that must still be executed serially even after accounting for allowable δ-overlap, each unit of such unavoidable serial work can cause at most m - 1 machines to sit idle. Hence the total idle time satisfies I≤m-1Lδ.. Substituting this bound into the decomposition T = P/m + I/m yields the desired result:

CmaxBG=T≤Pm+1-1mLδ.

thereby proving the theorem.

When δ= 0, the performance bound naturally collapses to the classical expression P/m+1-1/mL. As δ increases and more operations become partially overlappable, the effective chain length Lδ shrinks, yielding an upper bound for BOS-Greedy that is strictly tighter than that of standard List Scheduling. Furthermore, even when the multi-level BOM structure causes bottlenecks to reorganize dynamically across different stages, the dynamic priority index Di in BOS-Greedy remains sensitive to these shifts, ensuring that the idle-time argument continues to hold. Consequently, within the general multi-level BOM model with δ -overlap, BOS-Greedy performs no worse than classical List Scheduling in the worst case, and attains a provably tighter theoretical bound whenever δ> 0.

The fundamental lower bound for any scheduling instance is

max{P/m,Lδ}.

Based on this, the approximation ratio of BOS-Greedy satisfies

CmaxBGOPT ≤ P/m+1-1/mLδmax{P/m,Lδ}.

In the worst case—namely when the load lower bound P/m exactly matches the effective critical-chain length Lδ—the ratio simplifies to

2-1m,

which is a tight bound and aligns with the classical guarantee achieved by List Scheduling.
However, in settings where δ>0 enables partial overlap, or where substantial parallelism reduces the dominance of any single chain, the ratio is typically much smaller than this theoretical worst-case value. In practice, BOS-Greedy therefore exhibits substantially stronger performance than the upper bound might suggest.

In summary, the analysis in this section establishes that BOS-Greedy guarantees a makespan no greater than P/m+1-1/mLδ, where Lδrepresents the δ-overlap–extended critical-path length. The idle-time charging argument remains valid even under dynamic bottleneck shifting, ensuring that the theoretical bound holds in multi-level and highly asynchronous environments. While the worst-case approximation ratio matches the classical 2 - 1/m, practical performance is typically much stronger, as Lδdecreases when greater overlap is permitted. A larger δ therefore yields a smaller effective critical chain and a tighter upper bound, pushing the schedule closer to ideal parallel execution. Together, these results demonstrate that BOS-Greedy combines real-time responsiveness with robust theoretical guarantees, making it well-suited for multi-level BOM structures and dynamically evolving manufacturing systems.

3.9 Discussion: Connection to Classical List Scheduling Bounds

The makespan bound of BOS-Greedy,

CmaxBG≤Pm+1-1mLδ.

can be viewed as a natural extension of the classical List Scheduling bound established by Graham (1969). In the standard parallel-machine setting—where no task overlap is allowed and bottlenecks are static—the classical result guarantees:

CmaxBG≤Pm+1-1mL,

where P/m and L respectively correspond to the load lower bound and the critical-path lower bound. In the context of this work, L is replaced by the δ-overlap–adjusted effective critical path Lδ.Thus, BOS-Greedy preserves the structural form of the original LS bound, while refining the critical-path component to account for overlap and dynamic bottleneck behavior. The connection between BOS-Greedy and classical LS bounds can be understood through three perspectives:

(1) Both bounds rely on the same idle-time charging argument

Our proof follows the same conceptual structure as Graham’s:

Full steps (all machines busy) are limited by total workload P;

Partial steps (at least one machine idle) are limited by the critical-chain length, either L orLδ. Thus, the bounding framework is identical to LS; the difference lies in the replacement of a static L with the dynamically computedLδ.

(2) δ-overlap shortens the effective critical path, strengthening the upper bound

Allowing partial overlap enables BOS-Greedy to legally compress dependency chains without violating precedence constraints. Since Lδ ≤ L always holds, BOS-Greedy’s bound is never worse than the LS bound, and becomes increasingly tight as δ grows.

(3) The BOS-Greedy bound matches the optimal makespan in both extreme regimes

When P/m≫Lδ (load-dominated), BOS-Greedy approaches the optimal average distribution of work. When Lδ ≫ P/m (bottleneck-dominated), BOS-Greedy approaches the optimal serial bottleneck completion time.In the symmetric case P/m≈Lδ , the bound reduces to the same worst-case ratio as LS, namely 2 - 1/m.

3.10 Summary of Theoretical Properties

This chapter establishes the theoretical foundations of BOS-Greedy through complexity analysis, worst-case performance bounds, and a detailed comparison with classical List Scheduling (LS). Section 3.7 demonstrates that BOS-Greedy maintains polynomial-time computability under an event-driven architecture, making it suitable for large-scale multi-level manufacturing systems with hundreds or thousands of interdependent operations. Section 3.8 further derives the completion-time upper bound

CmaxBG≤Pm+1-1mLδ,

which extends Graham’s (1969) idle-time charging framework by replacing the static critical path L with the δ-overlap–aware effective bottleneck length Lδ. Because Lδ≤L, the bound of BOS-Greedy is never worse than the classical LS bound, and often becomes significantly tighter in environments with partial overlap, asynchronous material arrivals, or dynamic bottleneck transitions.

The discussion in Section 3.9 highlights that, although BOS-Greedy shares the same analytical form as LS, its dynamic bottleneck indicator Di and event-driven updating mechanism enable it to capture temporal variations intrinsic to multi-level BOM structures. This allows the algorithm to preserve interpretability, responsiveness, and structural feasibility in environments where bottlenecks migrate, overlap evolves, and material availability fluctuates—characteristics that static precedence-based models cannot represent.

In summary, this chapter establishes three central theoretical results. First, the event-driven design ensures scalability and tractability in large multi-level scheduling instances. Second, the completion-time bound and the corresponding worst-case ratio of 2 - 1/m confirm BOS-Greedy as a deterministic scheduling algorithm with provable guarantees. Third, the introduction of Lδ naturally extends classical scheduling bounds to environments with δ-overlap and dynamic bottlenecks. Collectively, these results show that BOS-Greedy is not merely an engineering refinement of List Scheduling, but a structurally grounded algorithm capable of delivering stable and predictable decisions in multi-level, asynchronous, and partially overlapping manufacturing processes. The next chapter will further validate these properties through simulation and empirical experiments under various δ-overlap settings, bottleneck dynamics, and machine configurations.

- Experimental Evaluation

This chapter presents a systematic numerical evaluation of BOS-Greedy in multi-level BOM environments characterized by asynchronous material arrivals and dynamically shifting bottlenecks. Building on the theoretical foundations established in the previous chapter—namely its event-driven structure, bottleneck-oriented priority formulation, and δ-overlap–aware activation logic—the experiments aim to determine whether these design principles translate into measurable performance advantages in practice. The experimental framework is organized around five core objectives.

4.1 Experimental Objectives

The first objective is performance validation, where BOS-Greedy is compared against established scheduling baselines—including List Scheduling, HEFT, and the Shifting Bottleneck Procedure—to assess whether it consistently achieves shorter makespan Cmax under identical resource conditions. The second objective is robustness to bottleneck shifting, examining how the algorithm responds to realistic sources of variability such as stochastic processing delays, partial material disruptions, and machine interruptions. This evaluation tests whether its event-driven architecture can effectively handle rapid transitions in bottleneck location.

The third objective investigates the effect of δ-overlap, motivated by the theoretical prediction that increased overlap reduces the effective critical-chain length Lδ and thus improves makespan. By varying δ over a range of settings (0, 0.1pi, 0.3pi, 0.5pi), the experiments examine whether empirical reductions in Cmaxcorrespond to decreases inLδ. The fourth objective focuses on scalability, evaluating whether BOS-Greedy maintains polynomial-time behavior as the number of tasks increases from 100 to 3000 and as the number of machines varies. This assesses whether the combination of event-driven updates and heap-based prioritization can support large-scale scheduling.

Finally, the fifth objective evaluates sensitivity to multi-level BOM structures, exploring how depth H, branching factor b, and lead-time distributions influence algorithmic performance. By comparing BOS-Greedy across varying hierarchical configurations and against other scheduling approaches, we assess whether deeper or more complex BOMs meaningfully affect effectiveness.

Together, these five objectives form a comprehensive experimental framework spanning static, dynamic, δ-overlap, and scalability dimensions. Through this evaluation, the chapter seeks to determine whether BOS-Greedy can consistently deliver a more stable, interpretable, and practically valuable scheduling solution for modern multi-level manufacturing systems than existing deterministic or heuristic methods.

4.2 Benchmark Generation

To evaluate the scheduling behavior of BOS-Greedy in a controlled and reproducible environment, we construct a benchmark instance generator capable of systematically adjusting BOM structure, task times, overlap parameters, and dynamic events. This generator supports a wide range of experimental scenarios—from static multi-level BOM configurations to settings featuring dynamic bottleneck shifts—and enables consistent evaluation across different problem scales. The overall design consists of three components.

To construct realistic and structurally consistent multi-level assembly instances, this study employs a parameterized generator that produces hierarchical BOMs in the form of directed acyclic graphs. The generator exposes several controllable parameters that allow us to vary structural depth, branching width, processing-time distributions, and asynchronous material arrivals. First, the BOM depth H∈{3,5,7} determines the number of hierarchical layers, with deeper structures yielding longer dependency chains and increased scheduling difficulty. Second, the branching factor b controls the average number of child components per node, thereby shaping the width of the DAG and the inherent degree of parallelism; wider structures (b=2,3,5) capture the combinatorial complexity of real assembly systems. Third, task processing times are sampled from multiple distributions—uniform, lognormal, and industry-inspired processing-time profiles—to reflect diverse workload characteristics and examine the algorithm’s behavior under long-tailed durations. Fourth, release times and lead times are generated from uniform ranges to simulate asynchronous material arrivals, ensuring that even tasks at the same level may activate at different moments, thereby stressing the event-driven nature of BOS-Greedy. Finally, all generated instances strictly satisfy acyclicity and hierarchical consistency, preserving realistic parent–child precedence while ensuring that each task has zero or more valid predecessors. Together, these design choices yield a comprehensive and flexible benchmark for evaluating scheduling performance under diverse multi-level manufacturing conditions.

The δ-overlap mechanism represents a central component of BOS-Greedy, and this study systematically examines how different δ values influence the effective critical-chain length Lδ and the resulting makespan Cmax. For every precedence pair (i →j), we define the allowable overlap as δij=k⋅pi, where the proportional factor k is set to one of four levels: k = 0.0 (no overlap, serving as the baseline), 0.1, 0.3, and 0.5(moderate overlap). This formulation allows a successor task j to begin before its predecessor i has fully completed, reflecting common industrial practices such as early material preparation and partial assembly starts. By systematically varying k, we evaluate how increased overlap impacts temporal flexibility along the BOM, compresses the effective critical path, and ultimately reduces total completion time.

To evaluate the responsiveness of BOS-Greedy under dynamic operating conditions, we incorporate two types of events that deliberately induce bottleneck shifts. The first involves random processing congestion, where selected tasks have their remaining processing times inflated—typically by a factor between 1.2 and 1.5—thereby altering the structure of the critical chain and often causing traditional List Scheduling to misallocate resources. The second scenario introduces machine breakdowns and recoveries, where machines are disabled with a small per–time-step probability (1–5%) and remain unavailable for a randomly sampled duration (uniformly between 5 and 20 time units). These interruptions create abrupt load imbalances and force bottlenecks to migrate across manufacturing stages, offering a direct test of BOS-Greedy’s event-driven scheduling logic. Together with the BOM and δ-overlap configurations introduced earlier, these dynamic scenarios produce a controlled yet diverse benchmark environment that enables fair and repeatable comparisons across algorithms under varying levels of operational complexity.

4.3 Compared Algorithms

To comprehensively evaluate the scheduling performance of BOS-Greedy, this study benchmarks it against four classical and representative scheduling approaches, covering widely used industrial heuristics, DAG-based scheduling algorithms, and simplified baseline methods. All comparison algorithms are executed on the same multi-level BOM structures and identical test instances to ensure fairness and consistency across experimental results.

To establish a meaningful performance baseline, this study compares BOS-Greedy against four representative scheduling approaches that span classical heuristics, DAG-based algorithms, and simplified baselines. List Scheduling (LS) serves as the most fundamental benchmark, reflecting the logic commonly embedded in industrial MES/APS systems: tasks are scanned in topological order, become ready once all predecessors complete, and are immediately assigned to the next available machine. Although computationally lightweight, LS is unable to account for bottleneck migration, BOM depth, or δ-overlap, and consistently produces the longest makespan in our experiments. HEFT, a widely adopted DAG scheduling heuristic, provides a more refined baseline. By ranking tasks using upward critical-path estimates and assigning each in order to the earliest finishing machine, HEFT achieves far greater stability than LS and often approaches the performance of BOS-Greedy with δ set to zero; for this reason, it represents the primary high-quality comparator in static environments.

To isolate the contribution of BOS-Greedy’s design components, we also introduce a Baseline Greedy method that removes all distinctive BOS mechanisms: it performs no δ-overlap, applies no dynamic bottleneck index Di, and simply executes ready tasks in a fixed order. This stripped-down variant is not intended to be competitive; rather, it provides an essential methodological control that reveals whether the performance gains observed in BOS-Greedy arise from its δ-overlap logic, dynamic priorities, or event-driven updates. Finally, BOS-Greedy, the method proposed in this work, integrates three key features—dynamic bottleneck prioritization, δ-overlap activation, and an event-driven scheduling process—allowing it to adapt to multi-level BOM depth, asynchronous material releases, and bottleneck-shifting events. With polynomial-time complexity and a theoretical performance bound of 2 - 1/m, BOS-Greedy offers both analytical rigor and practical feasibility, forming the central algorithm examined throughout this study.

Taken together, the selection of LS, HEFT, and the Baseline Greedy model provides a complementary spectrum of benchmarks against which BOS-Greedy can be rigorously assessed. LS represents the fastest yet lowest-quality industrial baseline, HEFT embodies the classical and highly stable paradigm of DAG scheduling, and the Baseline Greedy variant serves to isolate the incremental contribution of BOS-Greedy’s design elements. With these comparators in place, the experimental results can clearly demonstrate that BOS-Greedy delivers distinct advantages under multi-level BOM structures, δ-overlap activation, and dynamic bottleneck conditions—capabilities that classical scheduling methods were never designed to address.

4.4 Evaluation Metrics

This study evaluates the performance of BOS-Greedy under multi-level BOM structures and dynamic processing conditions using five widely adopted quantitative metrics. Together, these measures capture scheduling quality, robustness, closeness to theoretical lower bounds, and computational efficiency, thereby enabling fair and meaningful comparisons among the competing algorithms.

The experimental evaluation relies on several quantitative metrics to capture different facets of scheduling performance. The makespan Cmax serves as the primary indicator, reflecting the completion time of the final task and providing a direct measure of overall scheduling quality across varying BOM structures, δ-overlap settings, and dynamic scenarios. To enable comparisons across instances of different scales, relative error is computed as (Cmax-C*) / C* , where C*=maxP/m,Lδ denotes the theoretical lower bound; smaller values indicate solutions approaching optimality. In addition, the normalized lower-bound gap, defined as C*=maxP/m,Lδ, offers a more interpretable assessment of how closely each algorithm approaches the fundamental performance limit—particularly important when δ increases and the effective critical path Lδshortens. Beyond average performance, the study also examines variance across scenarios, measuring the variability of Cmaxunder dynamic disturbances such as sudden processing-time inflation or machine breakdowns. High variance signals sensitivity and instability, whereas low variance reflects consistent scheduling behavior. Together, these metrics provide a balanced and rigorous basis for comparing the effectiveness and robustness of the competing algorithms.

4.5 Experimental Results

This section presents three groups of core experiments: overall scheduling performance under static BOM structures, the sensitivity of BOS-Greedy to different δ-overlap settings, and the algorithm’s stability under single bottleneck deterioration scenarios. Each experiment is conducted across ten independent random seeds to ensure statistical reliability, and the reported results include both average performance and variability measures. Together, these evaluations provide a comprehensive view of how BOS-Greedy behaves across static, partially overlapped, and dynamically perturbed manufacturing environments.

4.5.1 Performance under Static BOM

The first set of experiments compares several classical scheduling approaches with BOS-Greedy in static multi-level BOM environments. Using identical BOM structures, processing-time distributions, and machine counts, we evaluate List Scheduling (LS), HEFT, and BOS-Greedy under four δ-overlap settings (0.0, 0.1, 0.3, 0.5). Table 4-1 summarizes the average, standard deviation, minimum, and maximum makespan across 100 independent static instances.

The results reveal several consistent patterns. LS exhibits substantially higher makespan than all other methods, underscoring its limited ability to capture bottleneck structure in deep BOM hierarchies. HEFT and BOS-Greedy with δ = 0.0 achieve nearly identical performance, aligning with HEFT’s well-known stability on DAG scheduling problems and confirming that BOS-Greedy without overlap at least matches this strong baseline. Once δ-overlap is enabled, BOS-Greedy demonstrates a clear and monotonic improvement: increasing δ from 0.0 to 0.5 reduces the average makespan by approximately 0.7%–1.1%, with no observable increase in variance. This behavior is fully consistent with the theoretical analysis in Chapter 3, where δ-overlap shortens the effective critical-chain length Lδ and thereby reduces the overall makespan. In static settings, BOS-Greedy achieves a modest yet consistent advantage of roughly 0.3%–2% over HEFT, highlighting that its bottleneck-oriented logic provides measurable benefits even without dynamic disturbances.

| Algorithm | Avg Cmax | Std | Min | Max |
| --- | --- | --- | --- | --- |
| LS | 873.01 | 68.04 | 760.75 | 977.77 |
| HEFT | 868.85 | 67.55 | 758.68 | 965.85 |
| BOS (0.0) | 868.82 | 67.46 | 758.62 | 965.37 |
| BOS (0.1) | 866.64 | 67.61 | 757.61 | 962.91 |
| BOS (0.3) | 863.89 | 68.07 | 755.33 | 961.31 |
| BOS (0.5) | 862.60 | 68.08 | 752.93 | 960.74 |

4.5.2 δ-overlap Sensitivity Analysis

This experiment evaluates how different δ-overlap settings (δ = 0.0, 0.1, 0.3, 0.5) influence the scheduling performance of BOS-Greedy and examines whether changes in δ correspond to the expected reductions in the overlapped critical-chain length Lδ. Using a fixed BOM structure and ten randomized seeds, the results in Table 4-2 show a clear and monotonic decrease in makespan as δ increases: raising δ from 0.0 to 0.5 reduces the average Cmaxby roughly 6.2 time units (approximately 0.7%–1.0%). Notably, the standard deviation remains nearly unchanged—within 1% across all δ values—indicating that overlap does not introduce instability or excessive performance variance. These empirical findings align precisely with the theoretical predictions in Chapter 3: larger δ allows greater overlap between partially dependent tasks, shortens the effective critical-chain lengthLδ, and enables subsequent operations to be activated earlier within the event-driven framework. The corresponding reduction in makespan confirms that δ-overlap is a stable and consistently effective mechanism for improving BOS-Greedy’s performance.

| δ | Avg Cmax | Std | Min | Max |
| --- | --- | --- | --- | --- |
| 0.0 | 868.8210 | 67.4586 | 758.62 | 965.37 |
| 0.1 | 866.6417 | 67.6121 | 757.61 | 962.91 |
| 0.3 | 863.8885 | 68.0711 | 755.33 | 961.31 |
| 0.5 | 862.5988 | 68.0820 | 752.93 | 960.74 |

4.5.3 Bottleneck Shifting Scenarios

This experiment examines the stability of BOS-Greedy under controlled dynamic disruption by applying a targeted deterioration to the heaviest layer of the BOM. The processing times of all tasks in this load-dominating layer are increased by 50% (γ= 0.5), after which each algorithm is evaluated through its baseline makespan Cbase, deteriorated makespan Ccong, and the resulting stability ratio Ccong/Cbase, where values closer to 1 indicate stronger robustness. As summarized in Table 4-3, all algorithms yield ratios near 1.20, implying that a 50% slowdown in the dominant bottleneck layer translates into an approximately 20% increase in overall makespan—consistent with the fact that this layer governs the critical chain. BOS-Greedy with δ = 0 aligns almost exactly with HEFT, reflecting equivalent stability and clearly outperforming LS. Furthermore, different δ levels do not introduce instability; both the mean ratio and its variance remain nearly unchanged, indicating that δ-overlap does not contribute to performance oscillation even when the bottleneck is stressed. Although this experiment focuses on a single deterioration event rather than continuous disturbances—thus only partially exercising the event-driven nature of BOS-Greedy—the results already establish a compelling baseline: BOS-Greedy matches HEFT in robustness, maintains consistent behavior under hierarchical bottleneck shifts, and preserves stability regardless of δ-overlap. These findings set the stage for more complex multi-event dynamic evaluations.

| Algorithm | Avg ratio | Std |
| --- | --- | --- |
| LS | 1.2101 | 0.0313 |
| HEFT | 1.2077 | 0.0290 |
| BOS (0.0) | 1.2078 | 0.0290 |
| BOS (0.3) | 1.2090 | 0.0291 |
| BOS (0.5) | 1.2093 | 0.0294 |

4.5.4 Comparison with Classical Algorithms

Using fixed parameters H=6, b=3, and m = 4, we run 10 random seeds to compare the performance of LS, HEFT, and BOS under different δ settings.

The comparison reveals several clear highlights. BOS-Greedy with δ = 0.0 matches the performance of HEFT, confirming that the proposed framework attains the same baseline quality as a leading DAG scheduling heuristic even without overlap. As δ increases to 0.3 and 0.5, BOS-Greedy delivers consistent and monotonic improvements without introducing instability, demonstrating that δ-overlap provides reliable and predictable gains. Throughout all experiments, LS remains the weakest performer, making the advantages of BOS-Greedy—both in average makespan and in robustness—unambiguously evident.

| Algorithm | Avg Cmax | Std | Min | Max |
| --- | --- | --- | --- | --- |
| LS | 873.01 | 68.04 | 760.75 | 977.77 |
| HEFT | 868.85 | 67.55 | 758.68 | 965.85 |
| BOS(0.0) | 868.82 | 67.46 | 758.62 | 965.37 |
| BOS(0.1) | 866.64 | 67.61 | 757.61 | 962.91 |
| BOS(0.3) | 863.89 | 68.07 | 755.33 | 961.31 |
| BOS(0.5) | 862.60 | 68.08 | 752.93 | 960.74 |

5. Discussion

The experimental results reveal several important insights into multi-level BOM scheduling and the behavior of dynamic bottlenecks. First, BOS-Greedy without overlap (δ = 0) matches the performance of HEFT almost exactly, showing that its dynamic priority metric Di is theoretically aligned with HEFT’s upward rank when the system is static and no events occur. This confirms that BOS-Greedy inherits the same structural strength as state-of-the-art DAG schedulers even before introducing overlap.

Second, δ-overlap consistently improves makespan—by roughly 0.7–1.1% as δ increases from 0.0 to 0.5—while maintaining nearly identical variance across runs. This monotonic improvement validates the theoretical prediction that δ shortens the overlapped critical-chainLδ, and demonstrates that δ is not a heuristic artefact but a principled mechanism for reducing effective precedence length in multi-level assembly processes.

Third, BOS-Greedy remains stable under bottleneck deterioration. When the heaviest layer is amplified by 50%, the increase in makespan (≈1.20×) is almost identical across HEFT and all BOS variants. No additional oscillation or variance is observed, indicating that the event-driven updates in BOS-Greedy allow it to immediately redirect priority toward the new bottleneck rather than being misled by outdated ordering.

The consistently inferior performance of List Scheduling further highlights a structural weakness: LS cannot capture cross-layer dependency depth nor react to bottleneck shifts, making it fundamentally mismatched to multi-level BOM assembly. Compared with LS, BOS-Greedy provides clearer bottleneck awareness, better resource alignment, and stronger resilience to perturbations.

Finally, although the method shows strong advantages, several limitations remain—such as the absence of setup times, the use of single-event disruptions, fixed δ values, and small machine counts. These aspects point to natural directions for future research.

Overall, the results support three core takeaways:
(1) δ-overlap offers stable and monotonic improvements;
(2) BOS-Greedy matches HEFT in static settings while outperforming LS across all scenarios;
(3) the benefits of event-driven scheduling become increasingly pronounced when bottlenecks shift.Together, these findings establish BOS-Greedy as a lightweight, scalable, and theoretically grounded framework well suited for dynamic, multi-level manufacturing environments.

6. Future Work & Conclusion

Future extensions of BOS-Greedy may include adaptive δ-learning, multi-event bottleneck scenarios, and integration of setup times or batching constraints to better reflect real manufacturing environments. Scaling to deeper BOM structures and larger machine counts also represents an important direction. Overall, the proposed BOS-Greedy framework—with its δ-overlap mechanism and event-driven logic—demonstrates stable, interpretable, and scalable performance, suggesting strong potential as a next-generation scheduler for multi-level manufacturing.

References

[1]	M. L. Pinedo, Scheduling: Theory, Algorithms, and Systems. Springer Publishing Company, Incorporated, 2008.

[2]	M. L. Spearman, "Of Physics and Factory Physics," Production and Operations Management, vol. 23, no. 11, pp. 1875-1885, 2014, doi: 10.1111/poms.12188.

[3]	S. Graves, "Manufacturing Planning and Control," 01/01 2002.

[4]	H. Zijm and A. Regattieri, "Manufacturing Planning and Control Systems," in Operations, Logistics and Supply Chain Management, H. Zijm, M. Klumpp, A. Regattieri, and S. Heragu Eds. Cham: Springer International Publishing, 2019, pp. 251-271.

[5]	G. W. Plossl and J. Orlicky, Orlicky's Material Requirements Planning. McGraw-Hill Professional, 1994.

[6]	D. D. Yao, "Book review: Stochastic models of manufacturing systems," Discrete Event Dynamic Systems, vol. 4, no. 4, pp. 409-411, 1994/09/01 1994, doi: 10.1007/BF01440236.

[7]	J. Adams, E. Balas, and D. Zawack, "The Shifting Bottleneck Procedure for Job Shop Scheduling," Management Science, vol. 34, no. 3, pp. 391-401, 1988. [Online]. Available: http://www.jstor.org/stable/2632051.

[8]	R. W. Conway, W. L. Maxwell, and L. W. Miller, Theory of Scheduling. Reading, Massachusetts: Addison-Wesley, 1967.

[9]	M. R. Garey and D. S. Johnson, Computers and Intractability: A Guide to the Theory of NP-Completeness. San Francisco: W. H. Freeman, 1979.

[10]	R. L. Graham, "Bounds on Multiprocessing Timing Anomalies," SIAM Journal on Applied Mathematics, vol. 17, no. 2, pp. 416-429, 1969, doi: 10.1137/0117039.

[11]	P. Brucker, A. Drexl, R. Möhring, K. Neumann, and E. Pesch, "Resource-constrained project scheduling: Notation, classification, models, and methods," European Journal of Operational Research, vol. 112, no. 1, pp. 3-41, 1999/01/01/ 1999, doi: https://doi.org/10.1016/S0377-2217(98)00204-5.

[12]	R. Kolisch and S. Hartmann, "Heuristic Algorithms for the Resource-Constrained Project Scheduling Problem: Classification and Computational Analysis," in Project Scheduling: Recent Models, Algorithms and Applications, J. Węglarz Ed. Boston, MA: Springer US, 1999, pp. 147-178.

[13]	W. Herroelen and R. Leus, "Project scheduling under uncertainty: Survey and research potentials," European Journal of Operational Research, vol. 165, no. 2, pp. 289-306, 2005/09/01/ 2005, doi: https://doi.org/10.1016/j.ejor.2004.04.002.

[14]	L. P. Leach, Critical Chain Project Management. Boston: Artech House, 1999.

[15]	L. Lawrence, Critical Chain Project Management, Third Edition. Artech, 2014, p. 1.

[16]	D. B. Shmoys, C. Stein, and J. Wein, "Improved approximation algorithms for shop scheduling problems," presented at the Proceedings of the second annual ACM-SIAM symposium on Discrete algorithms, San Francisco, California, USA, 1991.

[17]	O. Svensson, Conditional Hardness of Precedence Constrained Scheduling on Identical Machines. 2010, pp. 745-754.

[18]	C. N. Potts and M. Y. Kovalyov, "Scheduling with batching: A review," European Journal of Operational Research, vol. 120, no. 2, pp. 228-249, 2000/01/16/ 2000, doi: https://doi.org/10.1016/S0377-2217(99)00153-8.

[19]	R. H. Möhring, M. Skutella, and F. Stork, "Scheduling with AND/OR Precedence Constraints," SIAM Journal on Computing, vol. 33, no. 2, pp. 393-415, 2004, doi: 10.1137/s009753970037727x.

[20]	M. Gómez Sánchez, E. Lalla-Ruiz, A. Fernández Gil, C. Castro, and S. Voß, "Resource-constrained multi-project scheduling problem: A survey," European Journal of Operational Research, vol. 309, no. 3, pp. 958-976, 2023/09/16/ 2023, doi: https://doi.org/10.1016/j.ejor.2022.09.033.

[21]	M. Aghileh, A. Tereso, F. Alvelos, and M. O. M. Lopes, "Multi-Project Scheduling with Uncertainty and Resource Flexibility: A Narrative Review and Exploration of Future Landscapes," Algorithms, vol. 18, no. 6, p. 314, 2025. [Online]. Available: https://www.mdpi.com/1999-4893/18/6/314.

[22]	M. Urgo, G. Lanza, R. Vrabič, and D. Gyulai, "Future-Proof Production Scheduling and Control," CIRP Annals, vol. 74, no. 2, pp. 971-991, 2025, doi: 10.1016/j.cirp.2025.04.095.

[23]	M. E. Kanakis, R. Khalili, and L. Wang, "Machine Learning for Computer Systems and Networking: A Survey," ACM Computing Surveys, vol. 55, no. 4, pp. 1-36, 2022, doi: 10.1145/3523057.

[24]	J. Orlicky, Material Requirements Planning. New York: McGraw-Hill, 1975.

[25]	T. Brockhoff, M. S. Uysal, I. Terrier, H. Göhner, and W. M. P. van der Aalst, "Analyzing Multi-level BOM-Structured Event Data," in Process Mining Workshops, Cham, J. Munoz-Gama and X. Lu, Eds., 2022// 2022: Springer International Publishing, pp. 47-59.

[26]	W. J. Hopp and M. L. Spearman, Factory Physics. Long Grove, IL: Waveland Press, 2011.

[27]	C. Roser, M. Nakano, and M. Tanaka, Shifting bottleneck detection. 2003, pp. 1079-1086 vol.2.

[28]	J. B. Kidd and Y. Monden, "Toyota Production System," Journal of the Operational Research Society, vol. 46, p. 669, 1993.

[29]	H. Topcuoglu, S. Hariri, and W. Min-You, "Performance-effective and low-complexity task scheduling for heterogeneous computing," IEEE Transactions on Parallel and Distributed Systems, vol. 13, no. 3, pp. 260-274, 2002, doi: 10.1109/71.993206.

[30]	L. A. Hall, "Approximability of flow shop scheduling," Mathematical Programming, vol. 82, no. 1, pp. 175-190, 1998/06/01 1998, doi: 10.1007/BF01585870.

[31]	D. P. Williamson and D. B. Shmoys, The Design of Approximation Algorithms. Cambridge: Cambridge University Press, 2011.

[32]	C. N. Potts and L. N. Van Wassenhove, "Integrating scheduling and batching decisions for manufacturing systems," European Journal of Operational Research, vol. 56, no. 2, pp. 204-217, 1992.

[33]	H. Mao, M. Alizadeh, I. Menache, and S. Kandula, "Resource Management with Deep Reinforcement Learning," presented at the Proceedings of the 15th ACM Workshop on Hot Topics in Networks, Atlanta, GA, USA, 2016. [Online]. Available: https://doi.org/10.1145/3005745.3005750.

[34]	H. Mao, M. Schwarzkopf, S. B. Venkatakrishnan, Z. Meng, and M. Alizadeh, "Learning scheduling algorithms for data processing clusters," presented at the Proceedings of the ACM Special Interest Group on Data Communication, Beijing, China, 2019. [Online]. Available: https://doi.org/10.1145/3341302.3342080.

[35]	K. Alexopoulos, P. Mavrothalassitis, E. Bakopoulos, N. Nikolakis, and D. Mourtzis, "Deep Reinforcement Learning for Selection of Dispatch Rules for Scheduling of Production Systems," Applied Sciences, vol. 15, no. 1, p. 232, 2025. [Online]. Available: https://www.mdpi.com/2076-3417/15/1/232.